RTL proggress

REQUANT TEST:

=== REQUANT SANITY TEST START ===

in: Q(10.8)

out: Q(8.7)

Max pos in: 0.996094

Max neg in: -3.000000

Max pos out: 0.992188

Max neg out: -1.000000

VCD info: dumpfile waves.vcd opened for output.

ZERO

in: 0.000000

out: 0.000000

LSB DROP

in: 0.996094

out: 0.992188

NEGATIVE VALUE LSB DROP

in: -2.000000

out: -1.000000

POS CLIPPING

in: 1.000000

out: 0.992188

NEG CLIPPING

in: -2.000000

out: -1.000000

PASS THROUGH

in: 0.390625

out: 0.390625

--- SANITY TEST PASSED ---

SOP TEST

(cocotb) [ignaciobalbo] ~/serdes_digital_ffe/modules/fir_sop/work $ make sanity

--- [SANITY] Compiling fir_sop with Icarus Verilog ---

iverilog -g2012 -o ../verification/tests/fir_sop_sanity.out ../../sds_rx_ffe/design/rtl/sds_rx_ffe_pkg.sv ../design/rtl/fir_sop.sv ../verification/tests/fir_sop_vs_gold.sv

--- [SANITY] Running simulation ---

vvp ../verification/tests/fir_sop_sanity.out | tee ../verification/logs/fir_sop_sanity.log

------ STARTING SIMULATION ------

------ Comparing 101 acc values

VCD info: dumpfile waves.vcd opened for output.

--- TEST PASSED :) ---

--- N Cycles: 100

--- SCORE   : 100

../verification/tests/fir_sop_vs_gold.sv:54: $finish called at 1105000 (1ps)

--- [SANITY] Complete, test passed. Log: ../verification/logs/fir_sop_sanity.log –